ModbusPM

Protocol Macro

MODBUS for

CS1/CJ1 series

MAN STA42

Operation Manual

OMRON

Table of contents

1.	Specifications	3

2.	ModbusPM Master interface	4

2.1.	Principe	4

2.2.	Ladder program	5

2.3.	Read 1 bit (function 01 or 02)	6

2.4.	Read n words (function 03 or 04)	6

2.5.	Write 1 bit (function 05)	7

2.6.	Write 1 bit to N slaves	7

2.7.	Write 1 word (function 06)	8

2.8.	Write 1 word to N slaves	8

2.9.	Write n words to 1 slave (function 10)	9

2.10.	Write n words to N slaves	9

3.	ModbusPM slave interface	10

3.1.	Principe	10

3.2.	Operation	11

3.3.	PMSU configuration.	11

4.	Downloading the protocol macro	12

4.1.	program adaptations	12

4.2.	Flags and control bits of PMSU	14

# Specifications

ModbusPM is a protocol macro developed for CJ1/CS1 series providing master and slave Modbus interfaces. 
The following  functions codes are implemented :

01 :  Read n bit (output)
02 :	Read n bit (input)
03 :	Read n words (register)
04 :	Read n words (input)
05 :  Write 1 bit
06 :	Write 1 word
10 : 	Write n words

Write command (05-06-10) can be sent to N consecutive slave

The Slave interface needs a ladder program (ModbusPM.cxp) to prepare data according to the read/write master request. The Read/Write command are executed in the DM area

# ModbusPM Master interface

## Principe

MobusPM Master interface provide 10 sequences of read/write command.
Write to N commands can be sent to several slaves which should be consecutive.

Command should be specified through operand C2 of PMCR instruction.

| sequence | Designation | Operation |
| --- | --- | --- |
| 000 | 01 Read 1b | Read 1 bit |
| 001 | 02 Read 1b | Read 1 bit |
| 002 | 03 Read nW | Read n words |
| 003 | 04 Read nW | Read n words |
| 004 | 05 Write 1b | Write 1 bit |
| 005 | 05 Write to N | Write 1bit to N slaves |
| 006 | 06 Write 1W | Write 1 word |
| 007 | 06 Write to N | Write 1 word to N slaves |
| 008 | 10 Write nW | Write n words |
| 009 | 10 Writ to N | Write n words to N slaves |

## Ladder program

In case of no response or wrong response, the sequence is cancelled and Sequence End Flag does not change.

If you change the sequence number, be sure to use hexadecimal format in PMCR instruction.

## Read 1 bit (function 01 or 02)

|  |  |  | @PMCR(260) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C1 |  |  |  |
|  |  |  | #0 or #1 |  |  | Request |
| response |  |  | Source |  | S | 3 words |
| n+1 | D |  | Destination |  | S+1 | Slave number |
| data | D+1 |  |  |  | S+2 | Address |

Example :Read bit 0 from slave n°1 address 00AD.
 Logical port n°0, Port  RS232C n°2, Unit CJ1W-SCU41 n° 0 (SCU +10)

Request :	01 01 00AD 0001 6C2B
Response :01 01 01 01 9048

| Dest | Resp |  | Source | Request |  |
| --- | --- | --- | --- | --- | --- |
| W0 | 0002 | 2 words | D | 0003 | 3 words |
| W1 | 0001 | data | D1 | 0001 | slave n°1 |
|  |  |  | D2 | 00AD | address |

## Read n words (function 03 or 04)

|  |  |  | @PMCR(260) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C1 |  |  |  |
|  |  |  | #2 or #3 |  |  | Request |
| response |  |  | Source |  | S | 4 words |
| n+1 | D |  | Destination |  | S+1 | Slave number |
| Data 1 | D+1 |  |  |  | S+2 | Address |
| Data n | D+n |  |  |  | S+3 | Number of word |

Example :Read 2 words from slave n°1 address 00AD.
 Logical port n°0, Port  RS232C n°2, Unit CJ1W-SCU41 n° 0 (SCU +10)

Request : 01 03 00AD 0002 55EA
Response :01 03 0004 1111 2222 8E87

| Dest | Resp |  | Source | Request |  |
| --- | --- | --- | --- | --- | --- |
| W0 | 0003 | 3 words | D | 0004 | 4 words |
| W1 | 1111 | Data 1 | D1 | 0001 | slave n°1 |
| W2 | 2222 | Data 2 | D2 | 00AD | address |
| W3 | ---- |  | D3 | 0002 | 2 words |

## Write 1 bit (function 05)

| @PMCR(260) |  |  |  |
| --- | --- | --- | --- |
| C1 |  |  |  |
| #4 |  |  | Request |
| Source |  | S | 4 words |
| Destination |  | S+1 | Slave number |
|  |  | S+2 | Address |
|  |  | S+3 | 0000 or FF00 |

Example: Write to ON bit 00AD.00 of slave n°1.
 Logical port n°0, Port  RS232C n°2, Unit CJ1W-SCU41 n° 0 (SCU +10)

Request :	 01 05 00AD FF00 1DDB
Response : 01 05 00AD FF00 1DDB (Response similar to the request)

| Source |  |  |
| --- | --- | --- |
| D0 | 0004 | 4 words |
| D1 | 0001 | slave n°1 |
| D2 | 00AD | Address |
| D3 | FF00 | Write data |

Write data : FF00= ON  0000= OFF

## Write 1 bit to N slaves

| @PMCR(260) |  |  |  |
| --- | --- | --- | --- |
| C1 |  |  |  |
| #5 |  |  | Request |
| Source |  | S | 4 words |
| Destination |  | S+1 | Slave count |
|  |  | S+2 | Address |
|  |  | S+3 | 0000 or FF00 |

Example: write to ON bit 00AD.00 of slaves n°1,2 and 3.
 Logical port n°0, Port  RS232C n°2, Unit CJ1W-SCU41 n° 0 (SCU +10)

Request :	 01 05 00AD FF00 1DDB then 02 05 00AD FF00 1DE8 then 03….
Response : 01 05 00AD FF00 1DDB (Response similar to the request)

| Source |  |  |
| --- | --- | --- |
| D0 | 0004 | 4 words |
| D1 | 0003 | 3 slaves |
| D2 | 00AD | Address |
| D3 | FF00 | Write data |

Each response will be written to Destination area. If one of the slaves does not respond, sequence will be aborted. Repeat counter value will designate the slave in error.

## Write 1 word (function 06)

| @PMCR(260) |  |  |  |
| --- | --- | --- | --- |
| C1 |  |  |  |
| #6 |  |  | Request |
| Source |  | S | 4 words |
| Destination |  | S+1 | Slave number |
|  |  | S+2 | Address |
|  |  | S+3 | Write data |

Example: Write 1111 in to slave n°1 at address 00AD.
 Logical port n°0, Port  RS232C n°2, Unit CJ1W-SCU41 n° 0 (SCU +10)

Request :	 01 06 00AD 1111 D477
Response : 01 06 00AD 1111 D477 (Response similar to the request)

| Source |  |  |
| --- | --- | --- |
| D0 | 0004 | 4 words |
| D1 | 0001 | slave n°1 |
| D2 | 00AD | Address |
| D3 | 1111 | Write data |

## Write 1 word to N slaves

| @PMCR(260) |  |  |  |
| --- | --- | --- | --- |
| C1 |  |  |  |
| # 7 |  |  |  |
| Source |  | S | 4 words |
| Destination |  | S+1 | Slave count |
|  |  | S+2 | Address |
|  |  | S+3 | Write data |

Example :Write 1111 in to slaves n°1, 2 and 3 at address 00AD.
 Logical port n°0, Port  RS232C n°2, Unit CJ1W-SCU41 n° 0 (SCU +10)

Request :	01 06 00AD 1111 D477
Response :01 06 00AD 1111 D477 (Response similar to the request)

| Source |  |  |
| --- | --- | --- |
| D0 | 0004 | 4 words |
| D1 | 0003 | 3 slaves |
| D2 | 00AD | Address |
| D3 | 1111 | Write data |

Each response will be written to Destination area. If one of the slaves does not respond, sequence will be aborted. Repeat counter value will designate the slave in error.

## Write n words to 1 slave (function 10)

|  | @PMCR(260) |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 |  | S | 5 words |
|  | # 8 |  | S+1 | Slave number |
|  | Source |  | S+2 | Address |
|  | Destination |  | S+3 | Number of words to read |
|  |  |  | S+4 | Bytes count (S+3)x2 |
|  |  |  | S+5 | data 1 |
|  |  |  | S+6 | data 2 |
|  |  |  | S+n | data n |

Example : Write 1111 and 2222 in to slave n°1 at address 00AD.
Logical port n°0, Port  RS232C n°2, Unit CJ1W-SCU41 n° 0 (SCU +10)

Request :	01 10 00AD 0002 04 1111 2222 F5CE
Response :01 10 00AD 0002 D029

| Source |  |  |
| --- | --- | --- |
| D0 | 0007 | 7 words |
| D1 | 0001 | slave n°1 |
| D2 | 00AD | Address |
| D3 | 0002 | Number of words to write |
| D4 | 0004 | Bytes count |
| D5 | 1111 | Data 1 |
| D6 | 2222 | Data 2 |

## Write n words to N slaves

| @PMCR(260) |  |  |  |
| --- | --- | --- | --- |
| C1 |  | S | 5 words |
| # 9 |  | S+1 | slave count |
| Source |  | S+2 | Address |
| Destination |  | S+3 | Number of words to write |
|  |  | S+4 | Bytes count (S+3)x2 |
|  |  | S+5 | Data 1 |
|  |  | S+6 | Data 2 |
|  |  | S+n | Data n |

Example : write 1111 et 2222 in to slaves n°1,2 and 3 at address 00AD.
Logical port n°0, Port  RS232C n°2, Unit CJ1W-SCU41 n° 0 (SCU +10)

Request :	01 10 00AD 0002 04 1111 2222 F5CE
Response :01 10 00AD 0002 D029

| Source |  |  |
| --- | --- | --- |
| D0 | 0007 | 7 words |
| D1 | 0003 | 3 slaves |
| D2 | 00AD | Address |
| D3 | 0002 | Number of words |
| D4 | 0004 | Bytes count |
| D5 | 1111 | Data 1 |
| D6 | 2222 | Data 2 |

# ModbusPM slave interface

## Principe

ModbusPM slave provide 2 sequences :

- Read & Write 
Functions 03-04-06-10h are implemented with the following restriction :
The write n word commands (10h) checksum is not checked.

- Write with CRC
This sequence allow only one function: write n word command (10h) but CRC is controlled.

The reason of such division is that sequence Read & Write use a matrix to compare the requests to identify the function then execute the appropriate step. Matrix case works only in these cases :

- similar Header and terminator.
2- header and size information in the request.
3- header and same size.

Modbus protocol does not use terminator, and most function do not use bytes count information. The above case 3 could be used but the size of the request such Write n word (Function code 10h) are bigger than others Functions. This is why function 10h is treated in two steps. The frame is truncated by the matrix first and the rest of the frame is read by another step which cannot use CRC control because one part of the frame is missing.

As soon as the PMSU receive 8 bytes, the request is compared in the matrix.
If the Function code is 03, 04 or 06 the request is executed by the CPU then answer is sent.
If the function code is 10, the rest of the request is still in the buffer of the PMSU.

Step 10_Wr_Data is executed to read the rest of the request.
This part contains  address, number of words, bytes count but also the 1st byte of the 1st data to be written. The ladder program recovers this bytes and  place it into the MSB of word W4. The rest of the frame is written starting from LSB of W4 ( "~" before (W(4),1) ).

## Operation

Unlike the master interface, Slave interface should be assisted by a ladder program to prepare data from/to the address specified by the request. 
If you only need to use function 10h with a CRC control, you must activate Task01 and de-activate task02.
Read/Write is executed into the DM area starting from 00000 to DM 32767 (7FFF) with a maximum of 96 words (60h) by command.

Common task store the slave number to HR1 which is used by C3 of PMCR. The other tasks transfer the data to the address specified by the master request. When data are ready, the “Wait Release” flag is activated by the task to allow the PMSU to continue the execution of the sequence. Only one task should be activated. Use the property windows to de-activate the task (01 or 02) according to the sequence used Read & Write or Write with CRC

Procedure :

- Adapt the ladder program ModbusPM.cxp to the PMSU used (see Ch. flags and control bits).

- Activate the Task  01 or 02.

- Specify the slave number in the common task.

## PMSU configuration.

|  | Serial Communications Boards |  | Serial Communications Unit |  |
| --- | --- | --- | --- | --- |
| Reference | CS1W-SCB21 | CS1W-SCB41 | CS1W-SCU21 | CJ1W-SCU41 |
| Port 1 | RS232C | RS232C | RS232C | RS-422A/485 |
| Port 2 | RS232C | RS-422A/485 | RS232C | RS232C |
| API compatibles | CS1H-CPU67/CPU66/CPU65/CPU64/CPU63 CS1G-CPUCPU45/CPU44/CPU43/CPU42 |  |  | CJ1G-CPU45 CPU44 |

Communication port and Unit number (for SCU unit) determine the content of  C1 of PMCR instruction.

# Downloading the protocol macro

## program adaptations

1- Specify the PLC type

2 specify the PMSU type

3 – Edit the communication Port settings (1 or 2) and select Protocol Macro and choose the correct format. (8 bits should be use for Modbus)

4- Connect then transfer the setup and the protocol (mastes or slave) into the PMSU.

## Flags and control bits of PMSU

The address of flags and bits used by the ladder program are depending of the PMSU used.
Each unit use 25 words in CIO memory.

For SCB unit, n = 1900
For SCU unit, n = CIO 1500 + ( 25 x Unit number )

Port n°1

| Symbol | Channel | Bit | SCB | SCU n°0 | SCU n°1 |
| --- | --- | --- | --- | --- | --- |
| P1_Wait_Release | n | 00 | 1900.00 | 1500.00 | 1525.00 |
| P1_Abort_Switch | n | 03 | 1900.03 | 1500.03 | 1525.03 |
| P1_Seq_End | n + 9 | 11 | 1909.11 | 1509.11 | 1534.11 |
| P1_PMCR_Exe | n + 9 | 15 | 1909.15 | 1509.15 | 1534.15 |
| P1_Case_0 | n + 12 | 15 | 1912.00 | 1512.00 | 1537.00 |
| P1_Case_1 | n + 12 | 01 | 1912.01 | 1512.01 | 1537.01 |
| P1_Case_2 | n + 12 | 02 | 1912.02 | 1512.02 | 1537.02 |
| P1_Case_3 | n + 12 | 03 | 1912.03 | 1512.03 | 1537.03 |
| P1_Step_8 | n + 13 | 08 | 1913.08 | 1513.08 | 1537.08 |
| P1_Step_9 | n + 13 | 09 | 1913.09 | 1513.09 | 1537.09 |
| Port n°2 |  |  |  |  |  |
| Symbol | Channel | Bit | SCB | SCU n°0 | SCU n°1 |
| P2_Wait_Release | n | 08 | 1900.08 | 1500.08 | 1525.08 |
| P2_Abort_Switch | n | 11 | 1900.11 | 1500.11 | 1525.11 |
| P2_Seq_End | n + 19 | 11 | 1919.11 | 1519.11 | 1534.11 |
| P2_PMCR_Exe | n + 19 | 15 | 1919.15 | 1519.15 | 1534.15 |
| P2_Case_0 | n + 22 | 00 | 1922.00 | 1522.00 | 1537.00 |
| P2_Case_1 | n + 22 | 01 | 1922.01 | 1522.01 | 1537.01 |
| P2_Case_2 | n + 22 | 02 | 1922.02 | 1522.02 | 1537.02 |
| P2_Case_3 | n + 22 | 03 | 1922.03 | 1522.03 | 1537.03 |
| P2_Step_8 | n + 23 | 08 | 1923.08 | 1523.08 | 1537.08 |
| P2_Step_9 | n + 23 | 09 | 1923.09 | 1523.09 | 1537.09 |